Instructions for Construction

The following table lists all materials one will need to build the Dome.

Fabricating the Electronics

The following materials are required to build the electronics:

Table F2: Materials Required for Construction of Electronics

| Material | Specifications | Quantity |
| --- | --- | --- |
| Push Buttons | 0.25’’ in diameter | 4 |
| Resistor | 2200 Ω | 4 |
| Arduino | Arduino Uno Board | 1 |
| LED Light | Color-Changing RGB | 4 |
| LED Screen | I2C | 1 |
| Microphone | SparkFun Product #BOB-09964 | 1 |

The following tools and materials will also be necessary for construction:

- Soldering Iron

- Wiring

- Written Arduino Code

- Using the 4 push buttons and 2200 Ω resistors, construct four copies of the circuit shown below (see Figure F9).

Figure F9: Button and Resistor Circuit

- Solder the color-changing LED lights together on a circuit board and connect them to the Arduino Uno board. (see Figure F10)

Figure F10: Color-changing LED assembly

- Connect the I2C LED screen to the Arduino Uno board.

- Connect the Microphone to the Arduino Uno serial input.

- Upload the given code to the Arduino through the attached USB cord.

- Connect the Arduino to a wall socket to power the device.

Fabricating the Outer Shell

Table F1: Materials Required for Construction of Shell

| Material | Specifications | Quantity |
| --- | --- | --- |
| Polycarbonate | .125’’x24’’x24’’ | 1 |
| Polystyrene | .125’’x18’’x36’’ | 2 |
| Screws | .125’’x1.5’’ | 4 |
| Nuts | Fit to screws | 4 |

The following tools and equipment are required to construct the dome:

- Heat Vacuum Former

- Bandsaw

- Drillpress

- File (diameter approx. 0.25’’)

- Sander

- Phillips head screwdriver

- Soldering kit

- Metal bowl (10.5’’ in diameter)

- Using the bandsaw, cut the polycarbonate piece to 19’’x21’’ so that it will fit the vacuum former.

- Using the metal bowl as a form, vacuum form the polycarbonate to the bowl. It should look similar to Figure F1.

Figure F1: Vacuum-formed polycarbonate

- Use the sandblaster to frost the plastic until one cannot see distinct figures through it (see Figure F2).

Figure F2: Sandblasting the polycarbonate

- Using the bandsaw, cut the vacuum formed piece as close to the edge as possible, and set aside this piece.

- Laser cut the polystyrene according to Figure F3:

Figure F3: Base plate outlines

- Throw out all extra pieces from laser cutting EXCEPT for the center square of piece 1.

- Stack plates 1-5 in numerical order, maintaining alignment.

- Attach 2 washers to each screw and put screws through each of the four screw holes. The heads of the screws should rest on plate 1 (see Figure F4), and the shaft of the screw should come out of plate 5.

Figure F4: Screw resting on plate 1

- Attach another nut to each screw, tightening as necessary, to hold together the base plate assembly .

- Drop the polystyrene assembly into the bottom of the vacuum formed polycarbonate, with plate 1 facing outward. Some of the polycarbonate will overhang the polystyrene (see Figure F5).

Figure F5: Polycarbonate shell overhanging polystyrene base plate

- Trace around the polystyrene assembly with a pencil.

- Sand the polycarbonate shell down to match the polystyrene assembly.

- Find the center of the top of the vacuum formed polycarbonate, and use a drill press to drill a 0.4’’ hole.

- Set the polycarbonate shell on its top. Using the small file, file a single spot horizontally to create a 0.25’’ deep U-shaped cutout along the bottom of the shell (see Figure F6).

Figure F6: Shell cutout with power cord passing through

- Laser cut the second sheet of polystyrene according to Figure F7:

Figure F7: Box plate outlines

- Drill a hole approximately .5’’ in diameter in the center of piece 1.

- Place the cut pieces together, with pieces 2, 3, 4, and 5 forming the sides of a box, 1 forming the top, and 6 serving as the bottom.

- Glue the pieces together (see Figure F8).

Figure F8: Electronics cover box

- Insert the electronic assembly into the box (instructions for construction of this assembly are detailed below) and carefully thread the wires through the hole in the top.

- Secure the buttons and LED screen to the base plate.

- Glue the box to the interior of the base plate.

- Place the base plate assembly along the opening of the shell, with plate 1 facing outward

- Secure the base plate assembly to the polycarbonate shell using glue.